Sreya Kalyan

March 8, 2024

Prof. Keith Paarporn

CS 2160 – Mon/Wed (12:15pm)

Homework 3 – Document

Problem 1

.text

.globl f

f:

# Save the values of a0, a1, a2, a3 onto the stack

addi sp, sp, -16

sw a0, 0(sp)

sw a1, 4(sp)

sw a2, 8(sp)

sw a3, 12(sp)

# Call g(a, b)

mv a0, a0   # Pass parameter a

mv a1, a1   # Pass parameter b

jal ra, g   # Call function g

mv t0, a0   # Save the result in temporary register t0

# Call g(c + d)

add a0, a2, a3  # Calculate c + d and pass as parameter

mv a1, a0       # Pass parameter a0 (c + d) to function g

mv a0, t0       # Pass the result of the first call to g

jal ra, g       # Call function g

# Restore the values of a0, a1, a2, a3 from the stack

lw a0, 0(sp)

lw a1, 4(sp)

lw a2, 8(sp)

lw a3, 12(sp)

addi sp, sp, 16  # Restore the stack pointer

ret

Problem 2

- Little Endian Machine:

  - After First Instruction:

    - The value stored in memory location 0x214 is 0x88.

  - After Second Instruction:

    - The value stored in memory location 0x214 is 0x11.

- Big Endian Machine:

  - After First Instruction:

    - The value stored in memory location 0x214 is 0x11.

  - After Second Instruction:

    - The value stored in memory location 0x214 is 0x88.

Problem 3

- After Performing Shift by 4 and Bitwise OR Operation: 0x1234F678

- After Performing Shift by 4: 0x23456780

- After Performing Shift by 3 and Bitwise AND Operation: 0x00015454

Problem 4

- For Hexadecimal Value: 0x001080B3

  - Instruction Type: I-Type (Immediate Type)

  - Assembly Instruction: jalr, x1, x10, 0

- For Following Sequence: sw x5, 16(x30)

  - Instruction Type: S-Type (Store Type)

  - Hexadecimal Value: 0xF4846553

- For Instruction Fields: opcode = 0x33, funct3 = 0x0, funct7 = 0x20, rs2 = 5, rs1 = 7, rd = 6

  - Instruction Type: R-Type (Register Type)

  - Assembly Instructions: sub x6, x7, x5

  - Binary Version: 0000000 000101 00111 100 00000 0110011

Problem 5