Sreya Kalyan

February 5, 2024

CS 2160 – Mon/Wed (12:15pm)

Prof. Keith Paarporn

Assignment #2

- B[g] = A[f] + A[f + 1]

- # Load A[i]

slli x5, x28, 2

# x5 = i * 4 (Since each element of A occupies 4 bytes)

add x5, x5, x10        # x5 = &A[i]

lw x6, 0(x5)           # x6 = A[i]

# Load A[j]

slli x7, x29, 2

# x7 = j * 4 (Since each element of A occupies 4 bytes)

add x7, x7, x10        # x7 = &A[j]

lw x8, 0(x7)           # x8 = A[j]

# Add A[i] and A[j]

add x9, x6, x8         # x9 = A[i] + A[j]

# Store the result in B[8]

slli x28, x28, 2

# Convert i to the offset in bytes

addi x28, x28, 32

# Add an offset of 8 elements in array

# (8 * 4 bytes = 32 bytes)

add x28, x28, x11      # Calculate the memory address of B[8]

sw x9, 0(x28)          # Store the result (A[i] + A[j]) in B[8]

    - Loop 1 Instructions Executed: 3N + 1

    - Loop 2 Instructions Executed: 2N + 4

    - Loop 1:

for (int t1 = N; t1 != 0; t1--) {

s2 += 2;

}

    - Loop 2:

while (t1 > 0) {

s2 += 2;

t1--;

}

  - Code:

li      t1, 0            # Initialize I = 0

LOOP:

bge     t1, s1, DONE    # Exit loop if i >= a

sub     s1, s1, s2      # Subtract a-b

addi    t1, t1, 1       # Increment i

j       LOOP

DONE:

  - 32 Instructions Executed

  - Code:

li      t1, 0            # Initialize i to 0

LOOP1:

bge     t1, s1, DONE1   # Exit outer loop if i >= a

li      t2, 0            # Initialize j = 0

LOOP2:

bge     t2, s2, NEXT    # Exit inner loop if j >= b

slli    t3, t2, 2       # Calculate offset 4 * j

add     t3, t3, s3

# Calculate address: base address + offset

add     t4, t1, t2      # Calculate value i + j

sw      t4, 0(t3)       # Store value in D[4*j]

addi    t2, t2, 1       # Increment j

j       LOOP2

NEXT:

addi    t1, t1, 1       # Increment i

j       LOOP1

DONE1:

  - 54 Instructions Executed